AV-1900

vav.link/av-1900/

RS-232 CODES

Baud rate: 9600K
Stop Bit: 1
Delivery interval >80ms
No check digit

Default Codes:  ASCII  Command format: 97 45 45<CR><LF>

Table 1: Command Codes

| ASCII Code | Function |  | Remarks | Return codes |
| --- | --- | --- | --- | --- |
| 97 45 45 | ON | Turn on/ bring out of standby | Working | 97 20 20 01 |
|  |  |  | INPUT1 Volume XX=00-14 | 97 31 31 XX |
|  |  |  | INPUT2 Volume XX=00-14 | 97 32 32 XX |
|  |  |  | INPUT3 Volume XX=00-14 | 97 33 33 XX |
|  |  |  | INPUT4 Volume XX=00-14 | 97 34 34 XX |
|  |  |  | Master Volume XX=00-14 | 97 23 23 XX |
|  |  |  | Master Bass XX=00-0E | 97 24 24 XX |
|  |  |  | Master Treble XX=00-0E | 97 25 25 XX |
| 97 46 46 | OFF | Turn off | Standby | 97 20 20 00 |
| 97 47 47 | MUTE | Mute on/off | Mute Off = 0 / Mute = 1 | 97 21 21 0X |
| 97 43 43 | INPUT1+ | INPUT1 volume + | INPUT1 Volume XX=00-14 | 97 31 31 XX |
| 97 1B 1B | INPUT1- | INPUT1 volume - |  |  |
| 97 1D 1D | INPUT2+ | INPUT2 volume + | INPUT2 Volume XX=00-14 | 97 32 32 XX |
| 97 4E 4E | INPUT2- | INPUT2 volume - |  |  |
| 97 18 18 | INPUT3+ | INPUT3 volume + | INPUT3 Volume XX=00-14 | 97 33 33 XX |
| 97 1C 1C | INPUT3- | INPUT3 volume - |  |  |
| 97 11 11 | INPUT4+ | INPUT4 volume + | INPUT4 Volume XX=00-14 | 97 34 34 XX |
| 97 10 10 | INPUT4- | INPUT4 volume - |  |  |
| 97 1A 1A | BT_RESET | BT Disconnect | / | 97 26 26 00 |
| 97 15 15 | TREBLE+ | Increase treble gain by +1DB | Treble Gain XX=00-0E | 97 25 25 XX |
| 97 17 17 | TREBLE- | Decrease treble gain by -1DB |  |  |
| 97 19 19 | BASS+ | Increase bass gain by +1DB | Master Bass XX=00-0E | 97 24 24 XX |
| 97 16 16 | BASS- | Decrease bass gain by-1DB |  |  |
| 97 4A 4A | RESET | Reset all |  | 97 23 23 0A 97 25 25 07 97 24 24 07 97 31 31 0A 97 32 32 0A 97 33 33 0A 97 34 34 0A |
| 97 4B 4B | MAXVOL | Max Volume |  | 97 23 23 0A 97 25 25 07 97 24 24 07 97 31 31 0A 97 32 32 0A 97 33 33 0A 97 34 34 0A |
| 97 40 40 | VOL+ | Volume +/ out of mute | Master Volume XX=00-14 | 97 23 23 XX |
| 97 44 44 | VOL- | Volume -/out of mute |  |  |
| 97 12 12 | AUTOSTBY_ON | AUTO STANDBY ON | Default | 97 28 28 01 |
| 97 13 13 | AUTOSTBY_OFF | AUTO STANDBY OFF |  | 97 28 28 00 |
| 97 1E 1E | #WAKEUP_ON | STANDBY AUTO WAKE ON | Default, Shallow Standby | 97 35 35 01 |
| 97 1F 1F | #WAKEUP_OFF | STANDBY AUTO WAKE OFF | Deep Standby, Amp circuit off | 97 35 35 00 |
| 97 41 41 | BT_ON | BT_ON |  | 97 37 37 01 |
| 97 42 42 | BT_OFF | BT_OFF | Deactivate BT permanently | 97 37 37 00 |

Table 2: Direct Address Volume Codes

| ASCII | Function | Remarks | Return codes |
| --- | --- | --- | --- |
| 97 AX AX | X =0-F Set master volume to 0-15 | xx =01-14 | 97 23 23 xx |
| 97 BX BX | X =0-4 Set master volume to 16-20 |  |  |
| 97 CX CX | X =0-E Set Bass volume to 0-14 | xx =01 -0E | 97 24 24 xx |
| 97 DX DX | X =0-E Set Treble volume to 0-14 | xx =01 -0E | 97 25 25 xx |
| 97 EX EX | X =0-F Set INPUT1volume to 0-15 | xx =01-14 | 97 31 31 xx |
| 97 FX FX | X =0-4 Set INPUT1volume to 16-20 |  |  |
| 97 5X 5X | X =0-F Set INPUT2volume to 0-15 | xx =01-14 | 97 32 32 xx |
| 97 0X 0X | X =0-4 Set INPUT2volume to 16-20 |  |  |
| 97 6X 6X | X =0-F Set INPUT3volume to 0-15 | xx =01-14 | 97 33 33 xx |
| 97 7X 7X | X =0-4 Set INPUT3volume to 16-20 |  |  |
| 97 8X 8X | X =0-F Set INPUT4volume to 0-15 | xx =01-14 | 97 34 34 xx |
| 97 9X 9X | X =0-4 Set INPUT4volume to 16-20 |  |  |
| 96 AX AX | X =0-F Initial set master volume to 0-15 | xx =01-14 | 96 23 23 xx |
| 96 BX BX | X =0-4 Initial set master volume to 16-20 |  |  |
| 96 EX EX | X =0-F Initial set INPUT1volume to 0-15 | xx =01-14 | 96 31 31 xx |
| 96 FX FX | X =0-4 Initial set INPUT1volume to 16-20 |  |  |
| 96 5X 5X | X =0-F Initial set INPUT2volume to 0-15 | xx =01-14 | 96 32 32 xx |
| 96 0X 0X | X =0-4 Initial set INPUT2volume to 16-20 |  |  |
| 96 6X 6X | X =0-F Initial set INPUT3volume to 0-15 | xx =01-14 | 9633 33 xx |
| 96 7X 7X | X =0-4 Initial set INPUT3volume to 16-20 |  |  |
| 96 8X 8X | X =0-F Initial set INPUT4volume to 0-15 | xx =01-14 | 96 33 33 xx |
| 96 9X 9X | X =0-4 Initial set INPUT4volume to 16-20 |  |  |

Table 3:  Return Codes

- Response from Unit = 97 ** **  0X/XX 0A

- All responses end in \x0A or LineFeed

- Sample commands and responses

- Power Query – Command -97 20 20

- Power Query – Response – STANDBY        97 20 20 01\x0A = On/No Faults

- Power Query – Response – STANDBY        97 20 20 00\x0A = Off/Standby

| ASCII Code | Name | Function | Remarks | Return codes |
| --- | --- | --- | --- | --- |
| 97 20 20 | #STANDBY | Speaker working Status | Speaker OFF/Standby Speaker ON/Working | 97 20 20 00 |
|  |  |  |  | 97 20 20 01 |
| 97 21 21 | #MUTE | Mute On/Off | Mute Off = 0 Mute = 1 | 97 21 21 00 |
|  |  |  |  | 97 21 21 01 |
| 97 23 23 | Master_GAIN_GET | Volume Gain | 20 steps total XX=00-14 HEX code | 97 23 23 XX |
| 97 24 24 | Master_B_GAIN_ GET | Bass Gain | 14 steps total, XX=00-0E HEX code | 97 24 24 XX |
| 97 25 25 | Master_T_GAIN_ GET | Treble Gain | 14 steps total XX=00-0E HEX code | 97 25 25 XX |
| 97 26 26 | BT_BIT | Bluetooth status | BT unpaired = 0 BT paired , Pause= 1 BT paired, Play = 2 | 97 26 26 00 |
|  |  |  |  | 97 26 26 01 |
|  |  |  |  | 97 26 26 02 |
| 97 27 27 | BT_DEVICE | Bluetooth name | Length=0# Name= xxx HEX code | 97 27 27 0# xxx |
| 97 28 28 | AUTO_STANDBY | Auto standby Status | Auto standby OFF = 0 Auto standby ON = 1 | 97 28 28 00 |
|  |  |  |  | 97 28 28 01 |
| 97 31 31 | INPUT1_GAIN_GET | INPUT1 Volume Gain | 20 steps total XX=00-14 HEX code | 97 31 31 XX |
| 97 32 32 | INPUT2_GAIN_GET | INPUT2 Volume Gain | 20 steps total XX=00-14 HEX code | 97 32 32 XX |
| 97 33 33 | INPUT3_GAIN_GET | INPUT3 Volume Gain | 20 steps total XX=00-14 HEX code | 97 33 33 XX |
| 97 34 34 | INPUT4_GAIN_GET | INPUT4 Volume Gain | 20 steps total XX=00-14 HEX code | 97 34 34 XX |
| 97 35 35 | #AUTO_WAKEUP | Auto Wake Up status | Auto Wake Up OFF =0 | 97 35 35 00 |
|  |  |  | Auto Wake Up ON =1 | 97 35 35 01 |
| 97 37 37 | #BT_STATUS | BT Control Status | BT OFF = 0 | 97 37 37 00 |
|  |  |  | BT ON = 1 | 97 37 37 01 |
| 96 23 23 | #MASTER_INIT | Initial Setup MASTER Volume Gain | 20 steps total XX=00-14 HEX code | 96 23 23 XX |
| 96 31 31 | #INPUT1_INIT | Initial Setup INPUT1 Volume Gain | 20 steps total XX=00-14 | 96 31 31 XX |
| 96 32 32 | #INPUT2_INIT | Initial Setup INPUT2 Volume Gain | 20 steps total XX=00-14 | 96 32 32 XX |
| 96 33 33 | #INPUT3_INIT | Initial Setup INPUT3 Volume Gain | 20 steps total XX=00-14 | 96 33 33 XX |
| 96 34 34 | #INPUT4_INIT | Initial Setup INPUT4 Volume Gain | 20 steps total XX=00-14 | 96 34 34 XX |

NOTE: Auto-Standby activates after 30 minutes of mute or no operation. Any adjustment turns standby off.